PulseBlasterESR: High Speed Programmable

## Pulse Generator / Digital Word Generator

PulseBlasterESRTM is a high-speed, programmable, multi-channel pulse generator in the PCI or Compact-PCI form factor.  Based on the versatile PulseBlasterTM processor architecture, PulseBlasterESRTM offers up to 24 independently controlled channels, variable output rates, up to 4096 instruction words, and powerful program-flow features. Currently available clock frequencies are up to 400 MHz.

### Key Specifications

- High speed - currently up to 400 MHz clock frequency

- 24 independently controlled output channels (21 for -PRO); TTL 3.3 V (unterminated)

- Four BNC bracket mounted connectors, 50 ohm impedance matched

- Three IDC headers for 24 signal outputs

- Programmable pulse lengths from 2.5  ns (PRO, 400 MHz)  to 10 s per instruction

- Sub-1 ns rise/fall times

- 2.5 ns pulse resolution (at 400 MHz)

- 512 or 4096 (-PRO) pulse program instruction words

- Advanced pulse-program flow control: nested loops (8 levels), subroutines

- Software and hardware triggering; up to 10 MHz retriggering frequency

- PCI or c-PCI (PXI) form factor.

### Applications

Complex, high-speed, multi-channel pulse, pattern, and word generation. Customers’ applications have included

- Experiment and process control in Electron, Nuclear and Quadrupole Resonances (ESR, NMR, NQR)

- Mass Spectrometry and related test technologies

Ultrasound, radar, medical imaging

Sample Output

- Laser control in advanced communication

- Solid state physics experiments

- Nanomagnetism and spintronics applications

- Robotics and process synchronization

- Data bus emulation

General laboratory use.

(24 channels)

### Free Software Support

- Application Programming Interface (API) in C, for Windows and Linux

- Example programs in C

- Interfaces and examples - LabView, Matlab, IGOR-PRO, SpecMan, Visual Basic, Custom GUI.

### Hardware Warranty

- Standard warranty  - two years

- Longer warranty periods available

- Money-back guarantee – 30 days.

### Models and Ordering Information

#### PBESR-XX and PBESR-PRO-XX

XX – Clock frequency, in MHz.  Standard values are 200, 250, 300, 333, and 400 MHz.  Other values upon request.

Sample Part Number: PBESR-PRO-400 (400 MHz clock frequency, 2.5 ns shortest pulse, 4k memory depth).